Lab 3 - Serial Communication

Student: Mohammad Alghurbah  |  ID: 24043876

Req. v - Full System with Master Reset via Key 5

#include "mbed.h"

#include "arm_book_lib.h"

DigitalIn gasDetector(D2);

DigitalIn overTempDetector(D3);

UnbufferedSerial uartUsb(USBTX, USBRX, 115200);

Timer monitoringTimer;

bool gasAlarmState = OFF, tempAlarmState = OFF;

bool gasSimulationState = OFF, tempSimulationState = OFF;

bool continuousMonitoring = OFF;

void inputsInit() {

gasDetector.mode(PullDown);

overTempDetector.mode(PullDown);

}

void uartTask() {

char c = 0; char str[100]; int n;

if (uartUsb.readable()) {

uartUsb.read(&c, 1);

switch (c) {

case 1:

gasSimulationState = !gasSimulationState;

if (gasSimulationState)

uartUsb.write("WARNING: GAS DETECTED ", 23);

else uartUsb.write("Gas simulation OFF ", 20);

break;

case 2:

uartUsb.write(gasAlarmState ?

"GAS ALARM ACTIVE " : "GAS ALARM CLEAR ",

gasAlarmState ? 18 : 17);

break;

case 3:

uartUsb.write(tempAlarmState ?

"TEMP ALARM ACTIVE " : "TEMP ALARM CLEAR ",

tempAlarmState ? 19 : 18);

break;

case 4:

tempSimulationState = !tempSimulationState;

if (tempSimulationState)

uartUsb.write("WARNING: TEMPERATURE TOO HIGH ", 31);

else uartUsb.write("Temp simulation OFF ", 21);

break;

case 5:

gasSimulationState = tempSimulationState = OFF;

gasAlarmState = tempAlarmState = OFF;

uartUsb.write("ALARMS RESET ", 14);

break;

case 6:

continuousMonitoring = !continuousMonitoring;

if (continuousMonitoring) { monitoringTimer.start(); }

else { monitoringTimer.stop(); monitoringTimer.reset(); }

break;

}

}

if (gasDetector || gasSimulationState) gasAlarmState = ON;

if (overTempDetector || tempSimulationState) tempAlarmState = ON;

if (continuousMonitoring && monitoringTimer.elapsed_time() >= 2s) {

monitoringTimer.reset();

n = sprintf(str, "Gas: %s | Temp: %s ",

gasSimulationState ? "ACTIVE" : "CLEAR",

tempSimulationState ? "ACTIVE" : "CLEAR");

uartUsb.write(str, n);

}

}

int main() { inputsInit(); while(true) { uartTask(); } }