Lab 2 — Smart Home Security System

Student: Mohammad Alghurbah  |  ID: 24043876

Part 2 — Warning Mode (3 Fails → 30s Lockout)

#include "mbed.h"

DigitalOut led_green(PB_0);

DigitalOut led_red(PB_14);

DigitalIn btn0(D2, PullUp);

DigitalIn btn1(D3, PullUp);

DigitalIn btn2(D4, PullUp);

DigitalIn btn3(D5, PullUp);

DigitalIn btn4(D6, PullUp);

DigitalIn btn5(D7, PullUp);

const int secret_passcode[4] = {0, 1, 2, 3};

int entered_code[4] = {-1, -1, -1, -1};

int press_count = 0;

int failed_attempts = 0;

int get_button_press() {

if (btn0 == 0) return 0;

if (btn1 == 0) return 1;

if (btn2 == 0) return 2;

if (btn3 == 0) return 3;

if (btn4 == 0) return 4;

if (btn5 == 0) return 5;

return -1;

}

int main() {

led_green = 0;

led_red = 0;

while (true) {

int pressed_btn = get_button_press();

if (pressed_btn != -1) {

entered_code[press_count] = pressed_btn;

press_count++;

while (get_button_press() != -1) {

ThisThread::sleep_for(10ms);

}

ThisThread::sleep_for(150ms);

}

if (press_count == 4) {

bool is_correct = true;

for (int i = 0; i < 4; i++) {

if (entered_code[i] != secret_passcode[i]) {

is_correct = false;

break;

}

}

if (is_correct) {

led_green = 1;

ThisThread::sleep_for(2s);

led_green = 0;

failed_attempts = 0;

} else {

failed_attempts++;

if (failed_attempts >= 3) {

// Warning mode: slow blink for 30 seconds (15 x 2s cycles)

for (int j = 0; j < 15; j++) {

led_red = 1;

ThisThread::sleep_for(1s);

led_red = 0;

ThisThread::sleep_for(1s);

}

failed_attempts = 0;

} else {

led_red = 1;

ThisThread::sleep_for(2s);

led_red = 0;

}

}

press_count = 0;

}

ThisThread::sleep_for(10ms);

}

}