Lab 2 — Smart Home Security System

Student: Mohammad Alghurbah  |  ID: 24043876

Part 4 — Admin Override During Lockdown

#include "mbed.h"

DigitalOut led_green(PB_0);

DigitalOut led_blue(PB_7);

DigitalOut led_red(PB_14);

DigitalIn btn0(D2, PullUp);

DigitalIn btn1(D3, PullUp);

DigitalIn btn2(D4, PullUp);

DigitalIn btn3(D5, PullUp);

DigitalIn btn4(D6, PullUp);

DigitalIn btn5(D7, PullUp);

const int secret_passcode[4] = {0, 1, 2, 3};

const int admin_passcode[4]  = {5, 5, 5, 5};

int entered_code[4] = {-1, -1, -1, -1};

int press_count = 0;

int failed_attempts = 0;

int get_button_press() {

if (btn0 == 0) return 0;

if (btn1 == 0) return 1;

if (btn2 == 0) return 2;

if (btn3 == 0) return 3;

if (btn4 == 0) return 4;

if (btn5 == 0) return 5;

return -1;

}

int main() {

led_green = 0;

led_blue = 0;

led_red = 0;

while (true) {

int pressed_btn = get_button_press();

if (pressed_btn != -1) {

entered_code[press_count] = pressed_btn;

press_count++;

while (get_button_press() != -1) { ThisThread::sleep_for(10ms); }

ThisThread::sleep_for(150ms);

}

if (press_count == 4) {

bool is_correct = true;

for (int i = 0; i < 4; i++) {

if (entered_code[i] != secret_passcode[i]) { is_correct = false; break; }

}

if (is_correct) {

led_green = 1; ThisThread::sleep_for(2s); led_green = 0;

failed_attempts = 0;

} else {

failed_attempts++;

if (failed_attempts == 3) {

for (int j = 0; j < 15; j++) {

led_red = 1; ThisThread::sleep_for(1s);

led_red = 0; ThisThread::sleep_for(1s);

}

} else if (failed_attempts >= 4) {

led_red = 1;

Timer lockdown_timer;

lockdown_timer.start();

int admin_press_count = 0;

int admin_buffer[4] = {-1, -1, -1, -1};

int blink_counter = 0;

// Non-blocking lockdown loop with admin polling

while (lockdown_timer.elapsed_time() < 60s) {

led_blue = (blink_counter < 50) ? 1 : 0;

if (++blink_counter >= 100) blink_counter = 0;

int admin_btn = get_button_press();

if (admin_btn != -1) {

admin_buffer[admin_press_count++] = admin_btn;

while (get_button_press() != -1) { ThisThread::sleep_for(10ms); }

ThisThread::sleep_for(150ms);

}

if (admin_press_count == 4) {

bool is_admin = true;

for (int m = 0; m < 4; m++) {

if (admin_buffer[m] != admin_passcode[m]) { is_admin = false; break; }

}

if (is_admin) { failed_attempts = 0; break; }

else admin_press_count = 0;

}

ThisThread::sleep_for(10ms);

}

lockdown_timer.stop();

led_red = 0;

led_blue = 0;

if (failed_attempts >= 4) failed_attempts = 0;

} else {

led_red = 1; ThisThread::sleep_for(2s); led_red = 0;

}

}

press_count = 0;

}

ThisThread::sleep_for(10ms);

}

}