Lab 1 — Creating a Blinky LED

Student: Mohammad Alghurbah  |  ID: 24043876

Part 2 — Independent Blink Rates via Tickers

#include "mbed.h"

DigitalOut led_green(PB_0); // LED1 - Base speed (1s blink)

DigitalOut led_blue(PB_7);  // LED2 - Twice as fast (0.5s blink)

DigitalOut led_red(PB_14);  // LED3 - Half as fast (2s blink)

Ticker ticker_green;

Ticker ticker_blue;

Ticker ticker_red;

void toggle_green() { led_green = !led_green; }

void toggle_blue()  { led_blue  = !led_blue;  }

void toggle_red()   { led_red   = !led_red;   }

int main() {

// Toggle every half-period to achieve the desired blink rate

ticker_green.attach(&toggle_green, 500ms); // Blinks every 1s

ticker_blue.attach(&toggle_blue,   250ms); // Blinks every 0.5s

ticker_red.attach(&toggle_red,     1s);    // Blinks every 2s

while (true) {

ThisThread::sleep_for(1s); // Main loop idles - Tickers run in background

}

}